# Learning Notes

I built this ESP32 watering prototype by connecting the sensors, LCD screen, LED, motor driver, and water pump. I tested each part separately before combining them.

The main difficulty at the beginning was wiring. I connected something incorrectly, and the board and wires became hot. I unplugged the USB immediately, checked the connections, and corrected the wiring. After that, I learned to check power, ground, and signal wires carefully before turning on the system.

I first tried Arduino IDE, but I had software compatibility problems on my MacBook. I then changed to Thonny and MicroPython, which allowed me to connect to the ESP32 and run the program.

I also removed the buzzer module because it kept making noise during testing. It was distracting and was not necessary for the final system.

The soil sensor was the most important part to test. The values changed depending on the soil, sunlight, and where I placed the sensor. I needed a longer USB cable so that I could test soil away from my computer.

My approximate results were:

- Wet soil: 1000–1500

- Slightly dry soil: 1700–1800

- Very dry soil: around 2900

Based on these tests, I chose 2200 as the watering threshold. When the reading is 2200 or higher, the pump runs for three seconds. When the reading is below 100, the system shows `SENSOR CHECK` instead of watering.

I also tested the water-level sensor, but it was not reliable with my small water container, so I did not use it in the final automatic program. The pump worked, but I learned that the tube needed time to fill before water came out.

The final code was developed with programming guidance. My contribution was building the hardware, testing the components, recording and interpreting the readings, choosing the threshold, and adjusting the system based on the test results.