Smart Watering System – Test Results

ESP32 automatic watering project: calibration and final settings

Note: All numerical values are raw ADC readings (0–4095). They are not percentages, milliliters, or exact moisture levels.

| Test area | Observed reading / result | Interpretation and final setting |
| --- | --- | --- |
| Soil moisture sensor – wet soil | About 1000–1500 | Soil is moist enough. Do not water. |
| Soil moisture sensor – slightly dry soil | About 1700–1800 after around 7 hours in sunlight | Soil is becoming dry but is still acceptable. Do not water yet. |
| Soil moisture sensor – very dry soil | About 2900 | Soil is very dry. Watering is needed. |
| Automatic watering threshold | 2200 or higher | The final program starts the pump for 3 seconds, then waits 60 seconds before checking again. |
| Abnormal soil-sensor reading | Below 100 or repeatedly 0 | Usually means the sensor signal is disconnected or unstable. The system shows SENSOR CHECK and does not water. |
| Water-level sensor (tested separately) | Dry: 0; wet residue: about 960; first in water: 1500–1600; fully submerged: about 2600 | The sensor works, but it is not used in the final program because the small water cup gives unreliable low-water warnings. |
| Light sensor (tested separately) | Flashlight: 100–150; indoor light: about 1200; covered: about 4095 | Lower value means brighter light; higher value means darker light. |
| LCD and water pump | LCD I2C address: 0x27; pump and tubing tested successfully | LCD displays the soil value and status. The pump is controlled automatically by the ESP32. |
| DHT11 temperature/humidity sensor | ETIMEDOUT during test | The sensor did not respond, so it is not included in the final program. |